| 060. The transfer/separation section in the PRO 951 is the same as the PRO 1200 series. |  |
| --- | --- |
| Select the right answer |  |
|  | True |
|  | False |
| 065. In the vertical conveyance section, the shape of the vertical conveyance exit roller had been changed from an integrated type to a segmented type. Select the correct reason for it. |  |
| Select the right answer |  |
|  | To prevent paper wrap |
|  | To prevent mis-centering of paper |
|  | To reduce the weight of the roller |

| 050. What is the process line speed of the PRO 951? Select all correct answers. |  |
| --- | --- |
| Select the right answers |  |
|  | 330mm/s |
|  | 460mm/s |
|  | 290mm/s |
|  | 570mm/s |

| 047. For the PRO 951, the default setting of DIPSW42-5 is "ON". |  |
| --- | --- |
| Select the right answer |  |
|  | True |
|  | False |

| 039. Select 2 items which are newly added in the machine adjustment of the PRO 951. |  |
| --- | --- |
| Select the right answers |  |
|  | Printer Gamma Offset Auto. |
|  | CCD Right and Left Quality Adjustment |
|  | Fusing Speed Adjustment |
|  | Printer Centering Adjustment |

| 001. The PRO 951 is developed based on the PRO 1200 platform as a succession model to the PRO 950. |  |
| --- | --- |
| Select the right answer |  |
|  | True |
|  | False |

| 070. Select the correct reason why the fold position on the guide plate is optimized, or why the PET and the rib are added. |  |
| --- | --- |
| Select the right answer |  |
|  | To enhance the paper feed ability for thick paper |
|  | To enhance the paper feed ability for thin paper |
|  | To improve the ability for JAM processing |

| 034. Among all the [Quality Adjustment] items, which is the item you can look at the setting state on the top screen of the COPY tab? |  |
| --- | --- |
| Select the right answers |  |
|  | Density Setting |
|  | Sharpness |
|  | Background Removal |
|  | Original Type |
|  | Text/Photo |

| 045. Which item is displayed on the PRO 951 when DIPSW8-6 is changed to "1"? |  |
| --- | --- |
| Select the right answer |  |
|  | Custom Counter Threshold Set screen |
|  | Billing Total Counter |
|  | Tandem connection setting screen |

| 008. Regarding the rear side of the 951, the duct must be kept 200 mm away from the wall, because it is possible that packing may occur due to the solid toner. |  |
| --- | --- |
| Select the right answer |  |
|  | False |
|  | True |

| 020. The ErP function is a function to turn the main power switch OFF after a certain period of time, as a forced energy-saving mode. |  |
| --- | --- |
| Select the right answer |  |
|  | True |
|  | False |

| 058. Select all correct items as the improvement strategy for ozone air flow that the charging section of the PRO 951 incorporates. |  |
| --- | --- |
| Select the right answers |  |
|  | To install a new dedicated duct |
|  | To install a new ozone measurement sensor |
|  | To install a new fan |

| 006. Which of the following is true regarding the power supply capacity? |  |
| --- | --- |
| Select the right answer |  |
|  | The power supply capacity of the PRO 951 is the same as the PRO 950 and PRO 1200. |
|  | The power supply capacity of the PRO 951 is the same as the PRO 950. |
|  | The power supply capacity of the PRO 951 is the same as the PRO 1200. |

| 061. Select the correct reason why the waste toner box swing drive has been changed to the belt drive. |  |
| --- | --- |
| Select the right answer |  |
|  | To reduce the noise |
|  | To improve the accuracy of the swing operation |
|  | To reduce costs |

| 028. Select the correct description regarding the corner stapling angle when using the FS-532. |  |
| --- | --- |
| Select the right answer |  |
|  | The stapling angle cannot be changed. |
|  | You can select [Diagonal] or [Parallel]. |
|  | It is available for stapling at any angle. |

| 023. After the Tandem Job is completed, the machine state is maintained based on the setting to select whether to continue using or cancel the Tandem Setting. |  |
| --- | --- |
| Select the right answer |  |
|  | True |
|  | False |

| 011. The 951 IDC sensor must not be wiped with a dry cotton swab, because the toner gets stuck to the sensor and is unable to be removed due to the textured surface of the sensor. |  |
| --- | --- |
| Select the right answer |  |
|  | False |
|  | True |

| 025. Which is the choice of the Process Speed Setting? Select all that apply. |  |
| --- | --- |
| Select the right answers |  |
|  | Low Speed |
|  | Default |
|  | High Speed |
|  | Middle Speed |

| 003. Select all correct items regarding the product design and the plan policy. |  |
| --- | --- |
| Select the right answers |  |
|  | Realize the same high image quality as the PRO 1200 |
|  | Realize the space-saving body |
|  | Realize the low price for the system |
|  | Support office applications |
|  | Support ORU-M |

Оценка Для Ruslan Tishkov. Задание; Группа; Уровень; Баллов; Максимальное количество баллов; Процентов. 1; E_bizhubpro951_mainbody.060_1_2; English; Class 1; 1; 1; 100. 2; E_bizhubpro951_mainbody.065_3_2; English; Class 3; 1; 1; 100. 3; E_bizhubpro951_mainbody.050_2_2; English; Class 2; 1; 1; 100. 4; E_bizhubpro951_mainbody.047_2_2; English; Class 2; 1; 1; 100. 5; E_bizhubpro951_mainbody.033_2_2; English; Class 2; 1; 1; 100. 6; E_bizhubpro951_mainbody.039_2_2; English; Class 2; 1; 1; 100. 7; E_bizhubpro951_mainbody.001_2_2; English; Class 2; 1; 1; 100. 8; E_bizhubpro951_mainbody.070_4_2; English; Class 4; 1; 1; 100. 9; E_bizhubpro951_mainbody.034_4_2; English; Class 4; 1; 1; 100. 10; E_bizhubpro951_mainbody.045_3_2; English; Class 3; 1; 1; 100. 11; E_bizhubpro951_mainbody.008_2_2; English; Class 2; 1; 1; 100. 12; E_bizhubpro951_mainbody.020_1_2; English; Class 1; 1; 1; 100. 13; E_bizhubpro951_mainbody.058_1_2; English; Class 1; 1; 1; 100. 14; E_bizhubpro951_mainbody.006_2_2; English; Class 2; 1; 1; 100. 15; E_bizhubpro951_mainbody.061_1_2; English; Class 1; 1; 1; 100. 16; E_bizhubpro951_mainbody.028_2_2; English; Class 2; 1; 1; 100. 17; E_bizhubpro951_mainbody.023_2_2; English; Class 2; 1; 1; 100. 18; E_bizhubpro951_mainbody.011_2_2; English; Class 2; 0; 1; 0. 19; E_bizhubpro951_mainbody.025_3_2; English; Class 3; 0.5; 1; 50. 20; E_bizhubpro951_mainbody.003_3_2; English; Class 3; 1; 1; 100. Сумма; 18.5; 20; 92.5. Processing time: 00:29:42 Оценка по главам You have achieved 93 % of the points. 70 % were required. Therewith you have completed the training succesfully..